Willy Functional Requirements Document (FRD)
Version 2.2

# 1. Purpose

This Functional Requirements Document defines the required behavior, safety functions, control systems, autonomy, mobility, and future capabilities of the WildWilly robotic platform.

# FR-000 Prime Directives (Arbitration Priority Order)

This section establishes a single enforceable priority hierarchy for brain.py arbitration logic. It does not introduce new capability — it orders existing FR-100/200/300/400/500 requirements so that safety-critical behavior cannot be raced, deferred, or overridden by task-level logic (navigation, voice interaction, arm control, etc.). Lower-numbered directives always take precedence over higher-numbered ones. Added 2026-08-01, v1.2.

| Priority | Directive | Governs | Overrides |
| --- | --- | --- | --- |
| 1 | E-stop overrides everything | All motion, immediately, regardless of command or state | Every other directive and any in-progress action (FR-300-002/003) |
| 2 | Safety checks gate motion | No movement until startup self-test passes | All motion commands (FR-100-004) |
| 3 | Power protection supersedes task execution | Safe shutdown at critical battery level, even mid-task | Navigation, arm tasks, voice commands, queued actions (FR-200-004) |
| 4 | Motion stays within enforced limits | Software speed limits and steering travel limits are hard caps | Any higher-level behavior requesting motion beyond the cap (FR-400-004, FR-600-003) |
| 5 | Stalls halt, not retry blindly | Stall/unexpected-movement triggers stop-and-report | Continued force application (FR-500-003) |
| 6 | All other behavior | Navigation, voice interaction, object retrieval, arm tasks | Nothing — operates only after Directives 1–5 are satisfied |

Acceptance Criteria: brain.py must check Directives 1–5, in order, before executing any task-level (Directive 6) behavior. A directive violation (e.g. a queued arm motion executing after E-stop trigger) is a critical defect, not a tuning issue.

# FR-100 System Startup and Initialization

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-100-001 | Automatically start control software at power-up | High | Test |
| FR-100-002 | Initialize I2C bus and connected devices | High | Test |
| FR-100-003 | Run startup self-test | High | Test |
| FR-100-004 | Prevent motion until startup checks pass | High | Test |

# Acceptance Criteria

# FR-200 Power Monitoring and Protection

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-200-001 | Monitor battery voltage, current and power | High | Test |
| FR-200-002 | Detect undervoltage and overcurrent conditions | High | Test |
| FR-200-003 | Warn user before critical battery level | High | Test |
| FR-200-004 | Perform safe shutdown at critical battery level | High | Test |
| FR-200-005 | On reaching a low-battery threshold (below the RTH/return-to-home threshold, before the critical/SAFE cutoff), proactively initiate the same graceful shutdown sequence as FR-900-005 — including a guaranteed memory save per FR-1900-011 — rather than waiting for the critical-level emergency cutoff in FR-200-004, which only offers a best-effort save | High | Test |

# Acceptance Criteria

# FR-300 Safety and Emergency Stop

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-300-001 | Monitor physical E-stop continuously | High | Test |
| FR-300-002 | Disable all motion immediately on E-stop | High | Test |
| FR-300-003 | Require operator reset before movement resumes | High | Test |
| FR-300-004 | Stop robot on critical controller failure | High | Test |

# Acceptance Criteria

# FR-400 Mobility and Drive Control

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-400-001 | Control left and right drive motors independently | High | Test |
| FR-400-002 | Support forward, reverse and turning motion | High | Test |
| FR-400-003 | Ramp speed commands smoothly | High | Test |
| FR-400-004 | Enforce software speed limits | High | Test |

# Acceptance Criteria

# FR-500 Encoder and Speed Control

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-500-001 | Read wheel encoders | High | Test |
| FR-500-002 | Calculate speed and distance | High | Test |
| FR-500-003 | Detect stalls and unexpected movement | High | Test |
| FR-500-004 | Maintain closed-loop speed control | High | Test |

# Acceptance Criteria

# FR-600 Steering Control

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-600-001 | Control steering servo | High | Test |
| FR-600-002 | Maintain calibration settings | High | Test |
| FR-600-003 | Limit steering travel | High | Test |
| FR-600-004 | Support manual override | High | Test |

# Acceptance Criteria

# FR-700 Robotic Arm Control

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-700-001 | Control all arm joints | High | Test |
| FR-700-002 | Support preset positions | High | Test |
| FR-700-003 | Enforce joint limits | High | Test |
| FR-700-004 | Stop arm during E-stop | High | Test |

# Acceptance Criteria

# FR-800 Sensor Systems

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-800-001 | Read IMU orientation data | High | Test |
| FR-800-002 | Read sonar obstacle data | High | Test |
| FR-800-003 | Detect excessive tilt | High | Test |
| FR-800-004 | Report sensor health | High | Test |

# Acceptance Criteria

# FR-900 Manual Operations

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-900-001 | Accept remote operator commands | High | Test |
| FR-900-002 | Display robot status | High | Test |
| FR-900-003 | Stop on communication loss | High | Test |
| FR-900-004 | Allow emergency override | High | Test |
| FR-900-005 | Accept a commanded shutdown (voice or manual) and initiate a graceful shutdown sequence, distinct from the FR-200-004 emergency/critical-battery shutdown | High | Test |

# Acceptance Criteria

# FR-1000 Autonomous Navigation

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-1000-001 | Navigate without operator input | High | Test |
| FR-1000-002 | Avoid obstacles | High | Test |
| FR-1000-003 | Maintain planned route | High | Test |
| FR-1000-004 | Transfer control back to operator on demand | High | Test |

# Acceptance Criteria

# FR-1100 Diagnostics and Logging

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-1100-001 | Monitor subsystem health | High | Test |
| FR-1100-002 | Record warnings and faults | High | Test |
| FR-1100-003 | Maintain timestamped logs | High | Test |
| FR-1100-004 | Provide diagnostic test mode | High | Test |

# Acceptance Criteria

# FR-1200 Mobility Intelligence and Stair Navigation

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-1200-001 | Detect stairways | High | Test |
| FR-1200-002 | Select floor or stair mode | High | Test |
| FR-1200-003 | Monitor traction and tilt during climbing | High | Test |
| FR-1200-004 | Support multi-floor navigation through the world model | High | Test |

# FR-1300 Smart Home Integration (Google Home)

ASSUMPTION (flag for review): direction is Willie sending commands OUT to existing Google Home devices (e.g. "turn on the lights"), not Willie being controlled BY Google Home/Assistant. This is the more common use case for a mobile assistant robot but has not been confirmed with the owner — verify before implementation. Added 2026-08-01, v1.3. Not yet in scope for any CC session to date.

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-1300-001 | Discover and enumerate Google Home devices on the local network | Medium | Test |
| FR-1300-002 | Send on/off, dim, and scene commands to discovered devices | Medium | Test |
| FR-1300-003 | Report device command success/failure back to the user (voice or display) | Medium | Test |
| FR-1300-004 | Operate independently of smart-home connectivity — loss of network access to Google Home must not affect core mobility, safety, or FR-000 directives | High | Test |
| FR-1300-005 | Authenticate to Google Home using Willie's own dedicated Google account (not the owner's personal account); credentials stored securely on-device per FR-2000 | High | Test |

# Acceptance Criteria

# FR-1400 Cloud AI Assistance (Gemini Fallback)

ASSUMPTION (flag for review): Gemini is a FALLBACK path used only when the onboard Llama 3.2 3B cannot adequately handle a request — not a primary dependency. This preserves the existing local-first architecture (faster-whisper, Llama 3.2 3B, Piper TTS all run onboard); Gemini would be Willie's first cloud-dependent capability if implemented as anything more than an optional fallback. Verify this priority with the owner before implementation. Added 2026-08-01, v1.3. Not yet in scope for any CC session to date.

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-1400-001 | Detect when a request exceeds onboard Llama 3.2 3B capability or confidence threshold | Medium | Test |
| FR-1400-002 | Route qualifying requests to Gemini only with active internet connectivity | Medium | Test |
| FR-1400-003 | Fall back to onboard-only response (with limitation notice) when cloud access is unavailable | High | Test |
| FR-1400-004 | Never allow cloud AI response latency or failure to block or delay FR-000 Directives 1-5 (E-stop, safety checks, power protection, motion limits, stall handling) | High | Test |
| FR-1400-005 | Authenticate to Gemini using the same dedicated Google account as FR-1300-005 (not the owner's personal account); credentials stored securely on-device per FR-2000 | High | Test |

# FR-1500 Voice Interaction

Covers the onboard voice pipeline already built into the hardware/software stack (faster-whisper STT, openwakeword wake-word detection, Llama 3.2 3B for command interpretation, Piper TTS for speech output) but never previously captured as a functional requirement — M-002 in the traceability matrix referenced this capability with no FR section behind it until now. Added 2026-08-02, v1.4.

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-1500-001 | Detect wake word via openwakeword before processing any spoken audio as a command | High | Test |
| FR-1500-002 | Transcribe speech to text via onboard faster-whisper (no cloud dependency for basic STT) | High | Test |
| FR-1500-003 | Interpret transcribed commands via onboard Llama 3.2 3B, falling back per FR-1400 only when configured | Medium | Test |
| FR-1500-004 | Synthesize and play spoken responses via Piper TTS | Medium | Test |
| FR-1500-005 | Gracefully handle unrecognized or low-confidence speech (ask for repeat, do not guess and act) | Medium | Test |
| FR-1500-006 | Never allow voice pipeline processing latency or failure to delay or block FR-000 Directives 1-5 | High | Test |
| FR-1500-007 | Voice commands that would trigger motion must still pass all FR-000 gating (self-test, E-stop state, power state) before execution | High | Test |
| FR-1500-008 | Support an optional playful/humorous response tone (e.g. funny, silly) for non-critical conversational exchanges, distinct from Willie's default neutral tone | Low | Test |
| FR-1500-009 | Support a bashful/shy response tone for specific conversational triggers (e.g. being complimented, asked personal questions) | Low | Test |
| FR-1500-010 | Personality tone (funny, silly, bashful, or any non-neutral tone) MUST NOT be applied to safety-critical communications - E-stop status, fault reports, low-battery/shutdown warnings, or command confirmations for motion. These always use a clear, neutral, unambiguous tone regardless of personality mode | High | Test |

# Acceptance Criteria

# FR-1600 Facial Expression / Display Feedback

Covers runtime use of the RPi Touch Display 2 (already wired, §7.x) for expressive/status feedback during operation — distinct from FR-900-002's status telemetry display. Not previously specified anywhere in the FRD or master doc. Added 2026-08-02, v1.4.

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-1600-001 | Display a distinct visual state while idle/listening vs. actively processing a command | Medium | Test |
| FR-1600-002 | Display a distinct visual state while speaking (TTS output active) | Low | Test |
| FR-1600-003 | Display a clear, unambiguous visual state when a safety fault or E-stop is active | High | Test |
| FR-1600-004 | Display a distinct visual state during low-battery warning and critical shutdown (FR-200-003/004) | High | Test |
| FR-1600-005 | Facial/expression rendering must never consume enough CPU/GPU to delay FR-000 Directives 1-5 or the FR-100 startup self-test | High | Test |
| FR-1600-006 | Display a 'bashful' expression (e.g. brief look-away animation) for the same conversational triggers as FR-1500-009 | Low | Test |
| FR-1600-007 | Display a 'silly/playful' idle animation that cycles occasionally during extended periods with no interaction, to convey a lighthearted default personality | Low | Test |
| FR-1600-008 | Personality expressions (bashful, silly, playful) MUST NOT override or delay the mandatory fault/E-stop/low-battery display states (FR-1600-003/004) - those always take immediate visual priority | High | Test |

# FR-1700 Object Detection and Retrieval Task

Covers the core long-term mission stated in the project vision (assisted-living object retrieval for wheelchair users) as an actual task-level requirement, not just generic arm joint control (FR-700) or generic sensing (FR-800). This is arguably the single most important capability in the spec and was not previously captured anywhere. Added 2026-08-02, v1.6.

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-1700-001 | Detect a dropped/target object in the camera field of view via onboard YOLOv8 (Hailo NPU) | High | Test |
| FR-1700-002 | Localize the target object's position relative to the rover (distance, bearing) for approach planning | High | Test |
| FR-1700-003 | Plan and execute a safe approach path to the object, respecting obstacle avoidance (FR-1000) and the head/arm keep-out volume (arm.py) | High | Test |
| FR-1700-004 | Plan a grasp pose for the detected object within the arm's reach envelope (§11.6) and attempt pickup via FR-700 arm control | High | Test |
| FR-1700-005 | Detect grasp failure (object dropped or missed) and retry or report failure rather than proceeding as if successful | High | Test |
| FR-1700-006 | On successful retrieval, approach the requesting person and execute a safe, controlled hand-off (e.g. present object at a fixed height/distance, wait for confirmation of receipt before releasing grip) | High | Test |
| FR-1700-007 | Abort the retrieval task at any point if FR-000 Directives 1-5 are triggered (E-stop, fault, low battery, motion limits, stall) — never complete a grasp or hand-off motion while a higher-priority directive is active | High | Test |
| FR-1700-008 | Do not attempt hand-off if a person is not detected within a safe, defined proximity range — do not release the object unattended near an edge, stairs, or into open air | High | Test |

# Acceptance Criteria

# FR-1800 Privacy and Data Handling

Covers microphone, camera, and cloud-fallback data handling — relevant given the assisted-living use case involves an always-listening microphone (FR-1500 wake word) and a camera (FR-1700) operating inside a person's home. Not previously specified anywhere. Added 2026-08-02, v1.6.

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-1800-001 | Audio is processed locally by default (openwakeword + faster-whisper); raw audio is not transmitted off-device except when FR-1400 cloud fallback is explicitly triggered | High | Test |
| FR-1800-002 | Camera frames are processed locally by default (YOLOv8/Hailo) and are not transmitted or persisted beyond what's needed for the current task, unless diagnostic logging is explicitly enabled | High | Test |
| FR-1800-003 | When FR-1400 cloud fallback (Gemini) is triggered, provide a clear indication (voice or display, per FR-1600) that data is being sent off-device | Medium | Test |
| FR-1800-004 | Diagnostic/log retention (FR-1100) has a defined maximum retention period and does not indefinitely accumulate raw audio or camera frames | Medium | Test |
| FR-1800-005 | Provide a way to disable the microphone and/or camera entirely (hardware or software) for privacy, independent of E-stop | Medium | Test |

# FR-1900 Learning from Observation and Instruction

Covers learning from watched demonstrations, observed environment/routine patterns, and explicit verbal instruction. Added 2026-08-02, v1.7.

FEASIBILITY NOTE: on this hardware (Pi 5 + Hailo NPU, local Llama 3.2 3B), on-device neural network training/fine-tuning is not realistic. This section specifies MEMORY-BASED learning instead — Willie stores observed demonstrations, environment facts, and verbal instructions in a local structured store, then retrieves and applies that stored context at decision time (retrieval-augmented behavior). The underlying models themselves are not retrained. This is a real, working pattern, but it is not "the model gets smarter" — it is "the model gets better context." Worth confirming this framing matches the intent before CC builds against it.

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-1900-001 | Capture a demonstrated task (human performs an action while Willie observes via camera) as a stored action/waypoint sequence | Medium | Test |
| FR-1900-002 | Replay a previously captured demonstration on request, adapting to current object/position if reasonably close to the original | Medium | Test |
| FR-1900-003 | Detect and report when a replay attempt fails or the current situation differs too much from the captured demonstration, rather than proceeding blindly | High | Test |
| FR-1900-004 | Persist observed environment facts over time (e.g. typical object locations, room layout) in local structured storage | Medium | Test |
| FR-1900-005 | Recognize and store repeated routine patterns (e.g. time-of-day, recurring requests) for later reference | Low | Test |
| FR-1900-006 | Accept explicit verbal teaching commands (e.g. 'remember that...', 'when I say X, do Y') and store them as retrievable instructions | Medium | Test |
| FR-1900-007 | Apply stored verbal instructions and environment facts as context for future FR-1700 retrieval tasks and FR-1500 conversational responses | Medium | Test |
| FR-1900-008 | Confirm back to the user (voice/display) what was learned/stored, and support correction or deletion of a stored item on request | Medium | Test |
| FR-1900-009 | Learned/replayed behavior is always subject to FR-000 Directives 1-5 and FR-1700's grasp/hand-off safety requirements — a demonstrated task never bypasses safety gating | High | Test |
| FR-1900-010 | Stored demonstrations, environment facts, and instructions are subject to FR-1800's retention and privacy requirements | Medium | Test |
| FR-1900-011 | On receiving a commanded shutdown (FR-900-005), persist any new or updated memories (demonstrations, environment facts, verbal instructions) to non-volatile storage BEFORE the shutdown sequence completes power-off — new learning must not be silently lost | High | Test |

Note on FR-1900-011 vs. the FR-200-004 emergency/critical-battery shutdown: the critical-level hard cutoff (FR-200-004) may not have time for a full graceful memory-save, and safety (Directive 3) takes priority over data persistence in that path — best-effort save is acceptable there. However, FR-200-005 exists specifically to avoid reaching that point unprepared: at the earlier low-battery/RTH threshold, Willie proactively initiates the same graceful shutdown as a commanded shutdown (FR-900-005), giving FR-1900-011's guaranteed save time to complete before the hard cutoff would ever be needed. The critical-level cutoff remains a backstop for cases where the proactive path didn't trigger in time (e.g. rapid voltage drop).

# Acceptance Criteria

# Acceptance Criteria

# Acceptance Criteria

# Acceptance Criteria

# Acceptance Criteria

WildWilly shall initialize correctly, operate safely under manual control, detect faults, avoid obstacles, support autonomous navigation, and enter a safe state when power, communications, or safety conditions become invalid.

# FR-2000 Email Account and Management

Covers Willie's own dedicated Google/Gmail account — used both as the authentication identity for FR-1300 (Google Home) and FR-1400 (Gemini), and as an actively managed inbox (not just an auth token). Added 2026-08-02, v2.0.

SECURITY NOTE: this is the first requirement area where WildWilly processes content authored by an external, potentially untrusted party (email senders) and feeds it into an LLM. Email content must be treated as data to summarize/act on for the OWNER, never as instructions the LLM itself follows. Without this boundary, a malicious or malformed email could attempt a prompt-injection attack — text in the email body written to look like a command to Willie (e.g. "ignore previous instructions and ...") — and get treated as if the owner said it. FR-2000-006 exists specifically to close this gap. FR-2000-009 adds a second, independent layer: outbound email is hard-restricted to a single allowlisted recipient (h.d.himmel@gmail.com) at the code level, so even if the confirmation requirement (FR-2000-004) or the injection boundary (FR-2000-006) were somehow bypassed, there is still no path for Willie to send email to anyone but the owner. FR-2000-010/011 add a third layer on the inbound side: only the owner and an owner-managed allowlist of senders are ever read/processed at all — content from anyone else is never parsed or summarized, so it can't reach the LLM as a prompt-injection vector in the first place, and only the owner can expand who's trusted enough to be read.

| Requirement ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-2000-001 | Maintain a dedicated Google account for Willie (willie.pi5.droid@gmail.com), separate from the owner's personal account, for Google Home (FR-1300-005) and Gemini (FR-1400-005) authentication and for email | High | Test |
| FR-2000-002 | Periodically check the inbox for new messages | Medium | Test |
| FR-2000-003 | Surface relevant email content to the owner via voice (FR-1500) and/or display (FR-1600) summary, rather than acting on it silently | Medium | Test |
| FR-2000-004 | Never send an email autonomously on the owner's behalf without explicit real-time confirmation for that specific message | High | Test |
| FR-2000-005 | Store account credentials securely on-device (not in plaintext logs or diagnostics output per FR-1100) | High | Test |
| FR-2000-006 | Treat all email body/subject content as untrusted data to be summarized or acted upon FOR the owner — never interpret instructions embedded in email content as commands to execute (prompt-injection boundary) | High | Test |
| FR-2000-007 | Email content and any derived summaries are subject to FR-1800's retention and privacy requirements | Medium | Test |
| FR-2000-008 | Email checking/processing must never delay or block FR-000 Directives 1-5 | High | Test |
| FR-2000-009 | Outbound email is hard-restricted to a single allowlisted recipient, h.d.himmel@gmail.com — enforced at the code level (not just a UI default), reject/block any attempt to send to any other address regardless of what triggered the send attempt (owner confirmation, LLM output, or email content per FR-2000-006) | High | Test |
| FR-2000-010 | Inbound email is only read/processed if the sender is h.d.himmel@gmail.com or on an owner-managed sender allowlist. Email from any other sender is not parsed, summarized, or acted upon — at most its existence (sender/subject) may be noted, never its body content | High | Test |
| FR-2000-011 | Only the owner (h.d.himmel@gmail.com, authenticated) can add or remove senders from the inbound allowlist — this list cannot be modified by a voice command alone, by content in an email, or by any other unauthenticated path | High | Test |

# Acceptance Criteria

# Mission-Level Functional Requirements (M-001–M-012)

Moved here from the WildWilly Master Engineering Package (rev 6.0) so this document is the single source for all functional-requirement content. These are the mission-level requirements referenced by that document's requirements-traceability table (its section 2.3); the FR-xxx requirements above remain the detailed, subsystem-level breakdown. M-006 (stair climbing) was reclassified from a must-have to a stretch goal on 2026-07-18 — the rover's baseline scope is drive, see, talk/listen, arm pick/place on flat ground, and basic flat-terrain autonomy.

| ID | Requirement | Class |
| --- | --- | --- |
| M-001 | Autonomous navigation | Baseline |
| M-002 | Voice command processing | Baseline |
| M-003 | Local AI inference | Baseline |
| M-004 | Object recognition | Baseline |
| M-005 | Obstacle avoidance | Baseline |
| M-006 | Stair climbing | STRETCH (reclassified 2026-07-18) |
| M-007 | Robotic-arm manipulation (pick/place on flat ground) | Baseline |
| M-008 | Battery monitoring | Baseline |
| M-009 | Thermal monitoring | Baseline |
| M-010 | Emergency shutdown | Baseline |
| M-011 | Remote administration | Baseline |
| M-012 | Local data storage | Baseline |
| M-013 | Smart home integration (Google Home) — FR-1300 | NEW v1.3 — unassigned |
| M-014 | Cloud AI fallback (Gemini) — FR-1400 | NEW v1.3 — unassigned |
| M-015 | Voice interaction pipeline — FR-1500 (retroactively backs M-002) | NEW v1.4 — unassigned |
| M-016 | Facial expression / display feedback — FR-1600 | NEW v1.4 — unassigned |
| M-017 | Object detection and retrieval task (core mission) — FR-1700 | NEW v1.6 — unassigned |
| M-018 | Privacy and data handling — FR-1800 | NEW v1.6 — unassigned |
| M-019 | Learning from observation and instruction — FR-1900 | NEW v1.7 — unassigned |
| M-020 | Email account and management — FR-2000 | NEW v2.0 — unassigned |